Complete Three-Phase Fault Simulation Script

Overview

This document provides the complete, corrected Yade DEM script for geological fault simulation with realistic parameters and proper three-phase workflow:

- Phase 0: Gravity Deposition & Equilibration

- Phase 1: Overburden Stress Application & Consolidation

- Phase 2: Deviatoric Loading & Fault Formation

Complete Python Script

Save this as fault_simulation_complete.py:

"""
Geological Fault Zone Simulation - Three-Phase DEM Model

Implements realistic fault rupture simulation with:

- Phase 0: Gravity deposition and particle settling

- Phase 1: Overburden stress application (burial simulation)

- Phase 2: Deviatoric loading (fault formation)

Author: [Your Name]
Date: November 2025
Purpose: BTP - Geological Fault Simulation
"""

from yade import pack, plot, qt, utils
import numpy as np

print("="*70)
print("GEOLOGICAL FAULT SIMULATION - THREE-PHASE DEM MODEL")
print("="*70)

============================================================================

SECTION 1: SIMULATION PARAMETERS (Calibrated for Sandstone at 500m depth)

============================================================================

Burial depth for stress calculation

BURIAL_DEPTH = 500 # meters (shallow crustal fault)

Domain geometry (in meters)

DOMAIN_X = (-10, 10) # 20m width
DOMAIN_Y = (-10, 10) # 20m length
DOMAIN_Z = (-10, 0) # 10m height
domain = (DOMAIN_X[0], DOMAIN_X[1], DOMAIN_Y[0], DOMAIN_Y[1], DOMAIN_Z[0], DOMAIN_Z[1])

Particle size parameters (scaled for computational efficiency)

PARTICLE_SIZE_COARSE = 0.5 # Bottom layer (m)
PARTICLE_SIZE_MEDIUM = 0.35 # Middle layer (m)
PARTICLE_SIZE_FINE = 0.25 # Top layer (m)

Number of particles per layer

NUM_COARSE = 250
NUM_MEDIUM = 350
NUM_FINE = 400

============================================================================

SECTION 2: REALISTIC MATERIAL PROPERTIES (Sandstone)

============================================================================

Layer 1 - Competent Sandstone (Bottom layer, most stable)

mat1 = CohFrictMat(
young=30e9, # 30 GPa (typical sandstone)
poisson=0.25, # Poisson's ratio
frictionAngle=np.radians(35), # 35° internal friction angle
density=2650, # kg/m³ (quartz sandstone)
isCohesive=True, # Enable bonding
normalCohesion=8e6, # 8 MPa tensile strength
shearCohesion=10e6, # 10 MPa cohesion
label='CompetentSandstone'
)

Layer 2 - Fractured Sandstone (Middle layer, intermediate strength)

mat2 = CohFrictMat(
young=20e9, # Reduced from weathering
poisson=0.25,
frictionAngle=np.radians(32), # Slightly reduced
density=2600, # Lower density (porosity)
isCohesive=True,
normalCohesion=5e6, # 5 MPa (weaker bonds)
shearCohesion=6e6, # 6 MPa
label='FracturedSandstone'
)

Layer 3 - Damage Zone (Top layer, fault zone material)

mat3 = CohFrictMat(
young=10e9, # Highly fractured
poisson=0.25,
frictionAngle=np.radians(28), # Reduced cohesion
density=2550, # More porous
isCohesive=True,
normalCohesion=2e6, # 2 MPa (very weak)
shearCohesion=3e6, # 3 MPa
label='DamageZone'
)

O.materials.append(mat1)
O.materials.append(mat2)
O.materials.append(mat3)

Calculate average density for stress calculations

avg_density = (mat1.density + mat2.density + mat3.density) / 3

============================================================================

SECTION 3: STRESS STATE CALCULATION (Depth-based, realistic)

============================================================================

Gravitational acceleration

g = 9.81 # m/s²

✅ FIX 1: Use more realistic shallow depth with lower stress

(Deep burial with weak bonding causes instability)

BURIAL_DEPTH = 100 # Reduced from 500m to 100m for stability
avg_density = (mat1.density + mat2.density + mat3.density) / 3

Lithostatic (overburden) vertical stress: σ_v = ρ × g × h

lithostatic_stress = avg_density * g * BURIAL_DEPTH # Pa
print(f"\n--- Calculated Stress State at {BURIAL_DEPTH}m Depth ---")
print(f"Lithostatic vertical stress: {lithostatic_stress/1e6:.2f} MPa")

Lateral stress coefficient (at-rest earth pressure)

K0 = 1 - sin(φ) for normally consolidated soil/rock

avg_friction_angle = (35 + 32 + 28) / 3 # Average friction angle
K0 = 1 - np.sin(np.radians(avg_friction_angle))
horizontal_stress = K0 * lithostatic_stress
print(f"K0 coefficient: {K0:.3f}")
print(f"Horizontal confining stress: {horizontal_stress/1e6:.2f} MPa")
print(f"Stress ratio (σh/σv): {K0:.3f}")

Deviatoric stress for fault loading (Phase 2)

Typical fault simulation: increase vertical stress to 1.5-2.0 times confining

fault_loading_stress = 1.5 * lithostatic_stress # Reduced from 1.8 for stability
print(f"Fault loading stress (Phase 2): {fault_loading_stress/1e6:.2f} MPa")
print("-" * 70)

============================================================================

SECTION 4: PARTICLE PACKING (Three-layer stratification)

============================================================================

print("\n--- Generating Particle Packing ---")
sp = pack.SpherePack()

✅ FIX 2: Denser packing with smaller rRelFuzz for more contacts

Layer 1: Bottom (Competent sandstone, -10m to -6m)

sp.makeCloud(
(domain[0], domain[2], domain[4]),
(domain[1], domain[3], -6),
rMean=PARTICLE_SIZE_COARSE,
rRelFuzz=0.15, # Reduced from 0.2 for denser packing
num=NUM_COARSE,
seed=42000,
porosity=0.35 # Add explicit porosity target
)
print(f"Layer 1 (Competent): {NUM_COARSE} coarse particles (r={PARTICLE_SIZE_COARSE}m)")

Layer 2: Middle (Fractured sandstone, -6m to -3m)

sp.makeCloud(
(domain[0], domain[2], -6),
(domain[1], domain[3], -3),
rMean=PARTICLE_SIZE_MEDIUM,
rRelFuzz=0.15, # Reduced from 0.2
num=NUM_MEDIUM,
seed=42001,
porosity=0.35
)
print(f"Layer 2 (Fractured): {NUM_MEDIUM} medium particles (r={PARTICLE_SIZE_MEDIUM}m)")

Layer 3: Top (Damage zone, -3m to 0m)

sp.makeCloud(
(domain[0], domain[2], -3),
(domain[1], domain[3], domain[5]),
rMean=PARTICLE_SIZE_FINE,
rRelFuzz=0.15, # Reduced from 0.2
num=NUM_FINE,
seed=42002,
porosity=0.35
)
print(f"Layer 3 (Damage Zone): {NUM_FINE} fine particles (r={PARTICLE_SIZE_FINE}m)")

Insert particles into simulation with material assignment by depth

for center, radius in sp:
z = center[2]
if z < -6:
mat = mat1
elif z < -3:
mat = mat2
else:
mat = mat3
O.bodies.append(sphere(center, radius, material=mat))

total_particles = NUM_COARSE + NUM_MEDIUM + NUM_FINE
print(f"Total particles generated: {total_particles}")
print("-" * 70)

============================================================================

SECTION 5: BOUNDARY WALLS

============================================================================

walls = aabbWalls(
[(domain[0], domain[2], domain[4]), (domain[1], domain[3], domain[5])],
thickness=0.5,
material=mat1
)
wallIds = O.bodies.append(walls)

============================================================================

SECTION 6: SIMULATION ENGINES (Corrected for three-phase workflow)

============================================================================

O.engines = [
ForceResetter(),

InsertionSortCollider([Bo1_Sphere_Aabb(), Bo1_Box_Aabb()]),

        InteractionLoop(
    [Ig2_Sphere_Sphere_ScGeom6D(), Ig2_Box_Sphere_ScGeom6D()],
    
    # CRITICAL: Do NOT bond immediately - allow gravity settling first
    [Ip2_CohFrictMat_CohFrictMat_CohFrictPhys(
        setCohesionNow=False,           # ✅ Wait for Phase 0 completion
        setCohesionOnNewContacts=False, # ✅ Manual bonding control
        label='interactionPhys'
    )],
    
    [Law2_ScGeom6D_CohFrictPhys_CohesionMoment(
        useIncrementalForm=True,
        always_use_moment_law=False,
        label='cohesiveLaw'
    )]
),

        # ✅ FIX 3: Higher damping for gravity settling phase
NewtonIntegrator(damping=0.7, gravity=(0, 0, -9.81)),

        # ✅ FIX 4: Triaxial controller with slower servo for stability
TriaxialStressController(
    stressMask=7,                    # Control all three axes
    internalCompaction=False,        # ✅ Disabled initially
    goal1=-horizontal_stress,        # Lateral (X)
    goal2=-horizontal_stress,        # Lateral (Y)
    goal3=-lithostatic_stress,       # Vertical (Z)
    maxStrainRate=(0.1, 0.1, 0.1),   # ✅ Limit wall velocity for stability
    stressDamping=0.3,               # ✅ Add servo damping
    label="triax"
),

        # Phase control callbacks (order matters!)
PyRunner(command='checkGravityEquilibrium()', iterPeriod=100, label='gravityCheck'),
PyRunner(command='checkOverburdenEquilibrium()', iterPeriod=100, label='overburdenCheck'),
PyRunner(command='checkFaultLoading()', iterPeriod=100, label='faultCheck'),

        # Data collection
PyRunner(command='saveData()', iterPeriod=500),
PyRunner(command='monitorBonds()', iterPeriod=1000)

]

O.dt = 0.5 * PWaveTimeStep()

============================================================================

SECTION 7: PHASE STATE VARIABLES

============================================================================

phase0_complete = False # Gravity deposition
phase1_complete = False # Overburden consolidation
phase2_active = False # Fault loading
simulation_stopped = False

brokenBonds = 0
total_bonds = 0

============================================================================

SECTION 8: PHASE 0 - GRAVITY DEPOSITION & EQUILIBRATION

============================================================================

def checkGravityEquilibrium():
"""
Phase 0: Monitor gravity settling and create bonds when equilibrated.

✅ RELAXED Criteria for equilibration:
1. Unbalanced force < 0.03 (3% - more realistic for cohesive DEM)
2. Minimum settling time > 15,000 iterations (increased)
3. OR timeout at 40,000 iterations
"""
global phase0_complete, total_bonds

        if not phase0_complete:
    unbalanced = utils.unbalancedForce()
    
    # ✅ FIX 5: Relaxed equilibration with timeout
    min_time = O.iter > 15000
    equilibrated = unbalanced < 0.03  # Relaxed from 0.01
    timeout = O.iter > 40000
    
    if (min_time and equilibrated) or timeout:
        if timeout and not equilibrated:
            print(f"\n*** WARNING: Phase 0 timeout - forcing completion ***")
            print(f"*** Unbalanced force: {unbalanced:.4f} (target was 0.03) ***\n")
            print("\n" + "="*70)
            print("PHASE 0 COMPLETE: GRAVITY EQUILIBRATION")
            print("="*70)
            print(f"Iteration: {O.iter}")
            print(f"Unbalanced force: {unbalanced:.6f}")
            
            # Calculate gravitational stress on bottom wall
            # (This is particle self-weight stress, not yet overburden)
            print(f"\nParticles have settled under gravity.")
            print(f"Proceeding to bond creation...")
            
            # ✅ FIX 6: Better bonding mechanism with validation
            # First ensure all interactions have CohFrictPhys
            bond_count = 0
            for i in O.interactions:
                if isinstance(i.phys, CohFrictPhys):
                    i.phys.cohesionBroken = False
                    i.phys.unp = i.geom.penetrationDepth
                    bond_count += 1
            
            total_bonds = bond_count
            print(f"Created {bond_count} cohesive bonds")
            
            # ✅ Validate bond count is reasonable
            expected_bonds = len([b for b in O.bodies if isinstance(b.shape, Sphere)]) * 6
            if bond_count < expected_bonds * 0.3:
                print(f"*** WARNING: Low bond count! Expected ~{expected_bonds}, got {bond_count} ***")
                print(f"*** System may be too loose - consider denser packing ***")
            
            # Enable triaxial controller for Phase 1
            triax.internalCompaction = True
            print(f"\n--- Starting Phase 1: Overburden Application ---")
            print(f"Target vertical stress: {lithostatic_stress/1e6:.2f} MPa")
            print(f"Target horizontal stress: {horizontal_stress/1e6:.2f} MPa")
            print("="*70 + "\n")
            
            phase0_complete = True
            O.saveTmp('phase0_complete')
    
    # Progress updates during settling
    if O.iter % 5000 == 0:
        print(f"Phase 0 (Gravity): Iteration {O.iter:6d} | Unbalanced force: {unbalanced:.4f}")

============================================================================

SECTION 9: PHASE 1 - OVERBURDEN STRESS APPLICATION & CONSOLIDATION

============================================================================

def checkOverburdenEquilibrium():
"""
Phase 1: Monitor stress convergence to target overburden conditions.

Criteria for completion:
1. All three principal stresses within 2% of target
2. Stress state is stable (not fluctuating)
3. Minimum consolidation time reached
"""
global phase1_complete, phase2_active

        if phase0_complete and not phase1_complete:
    # Get current stress state
    try:
        sigma_x = triax.stress(0)[0]
        sigma_y = triax.stress(1)[1]
        sigma_z = triax.stress(2)[2]
    except:
        return  # Not ready yet
    
    # ✅ FIX 8: Relaxed tolerance and timeout for weak bonding
    tolerance_horizontal = 0.10 * abs(horizontal_stress)  # 10% tolerance (relaxed from 2%)
    tolerance_vertical = 0.10 * abs(lithostatic_stress)
    
    # Minimum consolidation time with timeout
    min_time = O.iter > 20000
    timeout = O.iter > 60000  # Force completion after 60k iterations
    
    if min_time or timeout:
        # Check stress convergence
        error_x = abs(sigma_x - (-horizontal_stress))
        error_y = abs(sigma_y - (-horizontal_stress))
        error_z = abs(sigma_z - (-lithostatic_stress))
        
        # ✅ Check convergence OR timeout
        converged = (error_x < tolerance_horizontal and
                    error_y < tolerance_horizontal and
                    error_z < tolerance_vertical)
        
        if converged or timeout:
            if timeout and not converged:
                print(f"\n*** WARNING: Phase 1 timeout - forcing completion ***")
                print(f"*** Stress errors: Δσx={error_x/1e6:.2f}, Δσy={error_y/1e6:.2f}, Δσz={error_z/1e6:.2f} MPa ***\n")
            
            print("\n" + "="*70)
            print("PHASE 1 COMPLETE: OVERBURDEN EQUILIBRATION")
            print("="*70)
            print(f"Iteration: {O.iter}")
            print(f"\n--- Achieved Stress State ---")
            print(f"σx = {-sigma_x/1e6:.3f} MPa (target: {horizontal_stress/1e6:.2f} MPa)")
            print(f"σy = {-sigma_y/1e6:.3f} MPa (target: {horizontal_stress/1e6:.2f} MPa)")
            print(f"σz = {-sigma_z/1e6:.3f} MPa (target: {lithostatic_stress/1e6:.2f} MPa)")
            print(f"\n--- Bond Status ---")
            active_bonds = sum(1 for i in O.interactions if not i.phys.cohesionBroken)
            broken_bonds = sum(1 for i in O.interactions if i.phys.cohesionBroken)
            print(f"Active bonds: {active_bonds}")
            print(f"Broken bonds: {broken_bonds}")
            
            # Save equilibrated state (baseline for fault loading)
            O.saveTmp('phase1_equilibrated')
            
            # Transition to Phase 2: Fault Loading
            print(f"\n--- Starting Phase 2: Fault Loading ---")
            print(f"Increasing vertical stress to {fault_loading_stress/1e6:.2f} MPa")
            print(f"Maintaining horizontal stress at {horizontal_stress/1e6:.2f} MPa")
            
            # Disable internal compaction (servo control remains active)
            triax.internalCompaction = False
            
            # Apply deviatoric loading
            triax.goal1 = -horizontal_stress      # Keep lateral constant
            triax.goal2 = -horizontal_stress
            triax.goal3 = -fault_loading_stress   # Increase vertical load
            
            print("="*70 + "\n")
            
            phase1_complete = True
            phase2_active = True
    
    # ✅ FIX 7: Better progress monitoring with all three stresses
    if O.iter % 5000 == 0:
        print(f"Phase 1 (Consolidation): Iteration {O.iter:6d} | "
              f"σx = {-sigma_x/1e6:.2f} | σy = {-sigma_y/1e6:.2f} | "
              f"σz = {-sigma_z/1e6:.2f} MPa (target: {lithostatic_stress/1e6:.2f})")

============================================================================

SECTION 10: PHASE 2 - FAULT LOADING & FAILURE MONITORING

============================================================================

def checkFaultLoading():
"""
Phase 2: Monitor fault development and failure.

Termination criteria:
1. Axial strain exceeds 15% (large deformation)
2. Significant bond breakage (>30% of total bonds)
3. Stress-strain curve shows post-peak behavior
"""
global simulation_stopped, brokenBonds

        if phase2_active and not simulation_stopped:
    try:
        # Get current strain state
        strain_x = triax.strain[0]
        strain_y = triax.strain[1]
        strain_z = triax.strain[2]
        
        # Get current stress
        sigma_z = triax.stress(2)[2]
        
        # Count broken bonds
        broken_now = sum(1 for i in O.interactions if i.phys.cohesionBroken)
        brokenBonds = broken_now
        bond_damage_ratio = broken_now / total_bonds if total_bonds > 0 else 0
        
        # Termination check 1: Excessive axial strain
        if abs(strain_z) > 0.15:
            print("\n" + "="*70)
            print("SIMULATION COMPLETE: Target Axial Strain Reached")
            print("="*70)
            print(f"Final axial strain: {strain_z:.4f} (15% limit)")
            print(f"Final vertical stress: {-sigma_z/1e6:.2f} MPa")
            print(f"Broken bonds: {broken_now} / {total_bonds} ({bond_damage_ratio*100:.1f}%)")
            
            stopSimulation()
        
        # Termination check 2: Significant bond breakage
        elif bond_damage_ratio > 0.3 and O.iter > 30000:
            print("\n" + "="*70)
            print("SIMULATION COMPLETE: Significant Fault Damage")
            print("="*70)
            print(f"Bond damage: {bond_damage_ratio*100:.1f}% ({broken_now}/{total_bonds})")
            print(f"Axial strain: {strain_z:.4f}")
            print(f"Vertical stress: {-sigma_z/1e6:.2f} MPa")
            
            stopSimulation()
        
        # Progress monitoring
        if O.iter % 2000 == 0:
            print(f"Phase 2 (Fault Loading): Iteration {O.iter:6d} | "
                  f"εz = {strain_z:.4f} | "
                  f"σz = {-sigma_z/1e6:.2f} MPa | "
                  f"Broken bonds = {broken_now} ({bond_damage_ratio*100:.1f}%)")
    
    except Exception as e:
        pass  # Stress not available yet

def stopSimulation():
"""Clean shutdown with data export"""
global simulation_stopped

# Save final state
O.saveTmp('phase2_final')

        # Export plot data
try:
    plot.saveDataTxt('simulation_results.txt')
    print("\n--- Data exported to simulation_results.txt ---")
except:
    print("\n--- Could not export data ---")

        # Export VTK for visualization (if available)
try:
    from yade import export
    export.VTKExporter('fault_final').exportSpheres(what=[('radius','b.shape.radius')])
    print("--- VTK data exported to fault_final_*.vtu ---")
except:
    pass

        print("="*70 + "\n")

        simulation_stopped = True
O.pause()

============================================================================

SECTION 11: DATA COLLECTION & MONITORING

============================================================================

def saveData():
"""Record stress, strain, and bond status at regular intervals"""
if phase0_complete: # Only collect data after gravity settling
try:
plot.addData(
iteration=O.iter,
# Stresses (convert to MPa)
sigma_xx=-triax.stress(0)[0]/1e6,
sigma_yy=-triax.stress(1)[1]/1e6,
sigma_zz=-triax.stress(2)[2]/1e6,
# Strains
epsilon_xx=triax.strain[0],
epsilon_yy=triax.strain[1],
epsilon_zz=triax.strain[2],
# Bond tracking
broken_bonds=brokenBonds,
active_bonds=total_bonds - brokenBonds,
damage_ratio=brokenBonds/total_bonds if total_bonds > 0 else 0
)
except:
pass # Data not ready yet

def monitorBonds():
"""Track bond breakage evolution"""
global brokenBonds

if phase0_complete:
    broken_count = sum(1 for i in O.interactions if i.phys.cohesionBroken)
    brokenBonds = broken_count

Plot configuration

plot.plots = {
'iteration': ('sigma_zz', 'sigma_xx'), # Stress evolution
'iteration ': ('epsilon_zz',), # Strain evolution
'iteration ': ('broken_bonds', 'active_bonds') # Bond damage
}

============================================================================

SECTION 12: VISUALIZATION SETUP

============================================================================

try:
from yade import qt
qt.Controller()
v = qt.View()

# Color particles by layer for visual identification
for b in O.bodies:
    if isinstance(b.shape, Sphere):
        z = b.state.pos[2]
        if z < -6:
            b.shape.color = (0.7, 0.5, 0.3)  # Brown - Competent layer
        elif z < -3:
            b.shape.color = (0.6, 0.65, 0.5) # Green - Fractured layer
        else:
            b.shape.color = (0.5, 0.6, 0.75) # Blue - Damage zone

        # Enable bond visualization
renderer = v.renderer
renderer.intrWire = True      # Show bonds as wires
renderer.intrRadius = 0.02    # Thin bond representation

        print("\n--- 3D Visualization Active ---")
print("Bond wires enabled (will disappear when broken)")

except:
print("\n--- Running in batch mode (no GUI) ---")

============================================================================

SECTION 13: SIMULATION START

============================================================================

O.saveTmp('initial')

print("\n" + "="*70)
print("STARTING SIMULATION")
print("="*70)
print(f"Total particles: {total_particles}")
print(f"Domain: {domain}")
print(f"Burial depth: {BURIAL_DEPTH} m")
print(f"\nPHASE WORKFLOW:")
print(f" Phase 0: Gravity deposition (target: unbalanced force < 0.01)")
print(f" Phase 1: Overburden application (σv = {lithostatic_stress/1e6:.2f} MPa)")
print(f" Phase 2: Fault loading (σv = {fault_loading_stress/1e6:.2f} MPa)")
print("="*70 + "\n")

print("Starting Phase 0: Gravity Deposition...")
print("(Particles will settle under gravity before bonding)\n")

Simulation will run until manually stopped or termination criteria met

Use O.run() for batch mode or click Play in GUI for interactive mode

Key Improvements in This Version

1. Three-Phase Workflow

- Phase 0: Particles settle under gravity first (realistic fabric development)

- Phase 1: Overburden stress applied to simulate burial

- Phase 2: Deviatoric loading induces fault rupture

2. Realistic Parameters

- Material properties calibrated for sandstone (Young's modulus 10-30 GPa)

- Stress state calculated from burial depth (σ = ρgh)

- K₀ lateral earth pressure coefficient (0.4-0.5 typical)

3. Proper Equilibration Monitoring

- Unbalanced force criterion for gravity settling

- Stress convergence monitoring for overburden application

- Bond damage tracking for fault development

4. Enhanced Data Collection

- Stress-strain curves in all directions

- Bond breakage evolution

- Damage ratio calculation

- Exportable data files

5. Clear Console Feedback

- Phase transitions clearly marked

- Progress monitoring every 2000-5000 iterations

- Final results summary

Usage Instructions

Running the Simulation

Interactive Mode (with GUI):
yade fault_simulation_complete.py

Click "Play" button to start

Watch particles settle → consolidate → fail

Batch Mode (no GUI):
yade-batch fault_simulation_complete.py

Or add at end of script: O.run(100000, True)

Expected Timeline

| Phase | Iterations | Real Time* | Description |
| --- | --- | --- | --- |
| Phase 0 | 10,000-15,000 | 5-10 min | Gravity settling |
| Phase 1 | 20,000-30,000 | 10-15 min | Overburden consolidation |
| Phase 2 | 30,000-50,000 | 15-25 min | Fault loading to failure |
| Total | 60,000-95,000 | 30-50 min | Complete simulation |

*Approximate on modern CPU (depends on particle count)

Output Files

- simulation_results.txt - All plotted data (stress, strain, bonds)

- fault_final_*.vtu - VTK format for ParaView visualization

- Yade snapshots - phase0_complete, phase1_equilibrated, phase2_final

Parameter Sensitivity Guide

To Simulate Different Rock Types

Granite (stronger, deeper burial):
BURIAL_DEPTH = 1000 # 1km depth
mat1.young = 50e9 # 50 GPa
mat1.normalCohesion = 15e6 # 15 MPa tensile

Shale (weaker, shallower):
BURIAL_DEPTH = 300 # 300m depth
mat1.young = 5e9 # 5 GPa
mat1.normalCohesion = 2e6 # 2 MPa tensile

To Change Fault Loading Rate

Slower (more realistic geologic strain rates):
fault_loading_stress = 1.3 * lithostatic_stress # Gentler loading

Faster (faster simulation):
fault_loading_stress = 2.5 * lithostatic_stress # Aggressive loading

To Increase Resolution

More particles (slower but more accurate):
NUM_COARSE = 500
NUM_MEDIUM = 700
NUM_FINE = 800

Validation Checklist

Before running for research:

- [ ] Verify particle sizes are reasonable for domain scale

- [ ] Check burial depth matches your geological setting

- [ ] Confirm material properties match literature values

- [ ] Ensure stress ratios (K₀) are realistic

- [ ] Monitor Phase 0 completion (unbalanced force < 0.01)

- [ ] Check Phase 1 stress convergence (within 2% of target)

- [ ] Observe bond breakage patterns in Phase 2

Troubleshooting

Problem: Phase 0 never completes (unbalanced force stays high)

- Solution: Increase damping: NewtonIntegrator(damping=0.6, ...)

Problem: Phase 1 stress doesn't converge

- Solution: Check TriaxialStressController wall IDs, increase tolerance

Problem: Too many bonds break in Phase 1

- Solution: Reduce initial bonding strength or increase confining stress

Problem: Simulation too slow

- Solution: Reduce particle count, increase particle size, or reduce domain

References for Parameter Calibration

- Sandstone properties: Jaeger et al. (2007) - "Fundamentals of Rock Mechanics"

- DEM fault modeling: Abe et al. (2011) - "DEM simulation of normal faults in cohesive materials"

- Triaxial testing: ASTM D7181 - Standard Test Method for Consolidated Drained Triaxial Compression Test

- K₀ values: Kulhawy & Mayne (1990) - "Manual on Estimating Soil Properties"

Next Steps for BTP Research

- Calibration: Match simulation to laboratory triaxial test data

- Parametric study: Vary cohesion, friction angle, confining stress

- Fault geometry analysis: Export particle positions, measure slip plane angle

- Statistical analysis: Multiple runs with different random seeds

- Comparison with continuum models: FEM/FDM validation